Rookwood Properties – THE ENCLAVE III Master Specification List

TRADEMARK UNITS & PUBLIC SPACES

3800 Enclave Ave. Cincinnati, OH   45241

Property Manager: Beth Westgerdes. (513)769-8888. Sales@TheEnclaveCincinnati.com

Date: 7-7-25

| Item | Product Information | Vendor | Contact | Photo |
| --- | --- | --- | --- | --- |
| Cabinets (Kitchen & Bathroom) | Kountry Wood Products – Mission Door Style, Flush Panel Door, Onyx Finish on Maple. (Or approved alternate.) Note: All base cabinets to be 34.5” high, except ADA units. All Bathroom Vanity Cabinets to be 21” deep x 34.5” high. Kitchen Plans to include an 18” wide trash pull-out cabinet with 2 bins in kitchen island. Include BZR Style Revolving Door Lazy Susan corner cabinet. Bathroom Vanities to have drawers. | Nisbet Brower | Chris Fischer (513)619-7107 Chris.fischer@nisbetbrower.com |  |
| Kitchen Countertops | Quartz: Nisbet Brower Stock Color: “Carrara Verna”. Kitchen Countertops. 3cm. Note: No quartz backsplash with tile backsplash, Sidesplash as needed. Include additional hole for soap dispenser. | Nisbet Brower | Chris Fischer (513)619-7107 chris.fischer@nisbetbrower.com |  |
| Kitchen Backsplash | Roca - Color Collection #U261, Tender Gray Matte Finish 3” x 6” x 6.3mm Subway Tile installed in horizontal brick pattern. Use 1/16” spacers. Grout – Mapei Ultracolor Plus FA, Color #02 Pewter. Matching caulk at countertop. On exposed vertial edge, use Schluter – Scheine Trim #AE60, Satin Anodized Aluminum, 6mm. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Cabinet and Drawer Pulls | Richelieu Bow Pull #BP16338195. (Brushed Nickel) 128 mm center to center Bow Pull, 6” overall dimension. | Tamarack/ Richelieu | Rick Kunkemoeller (513)978-3755 (cell) rkunkemoeller@richelieu.com |  |
| Kitchen Faucet | # Kohler –Simplice Pull Down Kitchen Faucet #K-596-CP. Polished Chrome Finish. Install Handle on Right Side. Single hole installation - do not use escutcheon plate. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Kitchen Sink | Omicron - Pompano Series Undermount Stainless Steel Sink #SNKFAB3118. 30” x 18-1/8”, Inside dimensions: 28” x 16-1/8” x 9”). Install in 33” min. sink base, 30” if sides are cut. | Nisbet Brower | Chris Fischer (513)619-7107 Chris.fischer@nisbetbrower.com |  |
| Kitchen Soap Dispenser | # Kohler – Contemporary Polished Chrome Soap Dispenser #K1895-C-CP. Refills from Top. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Dining Room Pendant Fixture | AFX - Cole 24” LED Pendant. #COLP24L30D1SN. 24” x 16” x 15” high. Satin Nickel. 3000K, 36W, 4200 Lumens. Install with bottom edge at 7’-3” above floor. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Kitchen, Hall, Closet & Shower Light | Kichler #43848WHLED30T “Utilitarian” Flush Mount Ceiling Light, 7.5” wide, 1-1/4” high, 15W LED, direct mount to junction box. 5 year warrantee. Wet rated for showers. Alternate: Globalux LDL2-6-15-120-9-30-WH. 6” dia., 3000K. 15W. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Ceiling Fan for ALL Bedrooms | Royal Pacific Fans – Europa IV #1044LED-1-WW-WW. Color: White. 15-7/8” high x 50” diameter, 12-1/2” to underside of blades, 3-Speed Reversible. WC-16 Wall Control. Install with stems so that underside of fan is at 8’0” above floor. | Ferguson | Rob Minardi 513-326-2986 cell: 513-484-5775 rob.minardi@ferguson.com |  |
| Pendant over Island /Sink | AFX Lighting – Twist LED Pendant #TWTP48L30D1SN. 48” l x 2” w x 1.5” high. Satin Nickel Finish, 3000K, 90 CRI. Install with bottom edge at 6’-9” above floor. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Dishwasher | GE – Stainless Steel Front Control Dishwasher #GDF510PSRSS. 23-3/4” w x 24” d x 34” h. | GE (Account #’s are specific to each Property) | PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Range | GE – Stainless Front, Freestanding Glass Top Electric Range #GRF500PZSS. 29-7/8” w x 28” d x 47”h. | GE (Account #’s are specific to each Property) | PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Range Cord | GE – 3 Wire, 4’, 40 Amp Cord #WX9X6. Or - 4 Wire, 4’, 40 Amp Cord #WX9X35. | GE (Account #’s are specific to each Property) | PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Microwave | GE – Stainless Steel Over-the-Range Microwave Oven #JNM3163RJSS. 29-7/8” w x 15-7/8” d x 16-7/16” h. | GE (Account #’s are specific to each Property) | PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Refrigerator | GE Stainless Steel Top Freezer, 17.5 cu. ft. Refrigerator with Icemaker. #GTS18GS. Note: Door comes with hinge on the left, if right swing is required order with code #M151 for installers to reverse the swing. 28” w x 30-1/2” d x 67-3/8” h. 30-5/8” wide with door open 90 degrees. No plastic water lines to icemakers – Use Copper or Pex Lines. | GE (Account #’s are specific to each Property) | PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Unit #3 ADA Refrigerator | GE – Stainless Steel Top Freezer, 19.2 cu. ft. Refrigerator with Icemaker #GIE19JSNRSS. Note: Door comes with hinge on the left, if right swing is required order with code #M151 for installers to reverse the swing. 29-3/4” w x 32-1/2” d x 66-3/8” high. 32-3/8” wide with door open 90 degrees. Preferable clear width: 33”-34”. No plastic water lines to icemakers – Use Copper or Pex Lines. | GE (Account #’s are specific to each Property) | PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Unit #3 ADA Range | GE – Stainless Front, Freestanding Glass Top Electric Range, #JB480SMSS. 30” w x 29” d x 47” h. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Unit #3 ADA Dishwasher | GE – Stainless Steel Front Control Dishwasher, #GDT225SSLS. 23-3/4” w x 24” d x 33-3/8” h. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Disposal | InSinkErator #503602, Badger 5, ½ HP Disposal. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Hot Water Heaters | 40 Gallon Low Boy Water Heater to meet Architectural Specs. |  |  |  |
| Bathroom Vanity Light Fixture | Modernforms - Ice #WS-57927-CH. 27” wide x 3” high. 3500K, 27W, 2400 Lumens. Install at 81” to center of mounting box. At 72” double bowl vanities, install (2) fixtures centered over sinks. At 60” double bowl vanities, install (1) 37” wide fixture centered over vanity. #WS-57937-CH. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Framed Bathroom Mirror | 28” x 34” high Non-Beveled Mirror in 2” Frame Profile #313., Metal Z-Clip attached for vertical mounting. Delivery available. Install at 75” to top of frame. Install 2 mirrors at 72” double bowl vanities. At 60” double bowl vanity, install (1) mirror - 52” wide x 34” high. | Artonomy | Carrie Jennings/ Jayne Menke 513-651-2787 carrie@artonomyinc.com |  |
| Wall Tile at Shower Surround. | Hamilton Parker - Atlas Concorde - Square One - Odyssey, Matte Finish. 12” x 24” x 8mm Tile installed in vertical grid pattern. 1/8” grout joint. Grout – Mapei Ultracolor Plus FA, Color #02 Pewter. Use matching caulk #593315, Keracaulk S- Pewter. White Silicone Caulk at Tub joint. Install Wall Tile up to Ceiling. Include Schluter - Schiene #AE-80, Satin Anodized Aluminum, edge band on vertical exposed tile edge. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Tile Shower Floor for ADA Unit #3 | Panaria Home - Gray Range Muretto Linear Mosaic, Mesh Mounted, 12” x 28”. Mapei Ultracolor Plus FA Grout, Color: #107 Iron. Install with Linear Slot Drain at showerhead end, Brushed Nickel Finish. Schluter Kerdi Shower Pan System. Provide tapered shower transition from LVT floor to tile shower floor at door. Construct per ADA requirements. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Vanity Countertops and Sinks | Nisbet Brower Stock Quartz - 2cm #3003 GJ Quartz. 4” Backsplash and Sidesplash. Drilled for single hole faucet. | Nisbet Brower | Chris Fischer (513)619-7107 chris.fischer@nisbetbrower.com |  |
| Bathroom Undermount Sink | # Omicron 18” x 13” Rectangular Undermount White Porcelain Sink. | Nisbet Brower | Chris Fischer (513)619-7107 chris.fischer@nisbetbrower.com |  |
| Bathtub Faucet & Showerhead | # Kohler – Honesty Shower Head # #K-TS99-764-4-CP, Polished Chrome. # #KP8304-KS, Rite Temp Rough-In Valve with Stops. (Shower Units Only) To be installed at 42” high to center of shower vlave. # Kohler – Honesty Shower Head # #K-TS99-764-4-CP, Polished Chrome. Install at 80” high. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Shower Doors (Full Size 5’ Showers) | Basco Infinity Series – #4500-60, 76” Tall (Overall height from base of track to top of track) frameless ¼” thick bi-pass doors, Clear Glass, Silver Anodized Aluminum Frame, 2 Towel Bars Standard. | Basco | Jacob Baldoc jbaldoc@bascoshowerdoor.com (513)283-9033 |  |
| ADA Shower Door. (Unit #3) | Basco Celesta Frameless 3/8” Swinging Door with Fixed Panel. #CELN-935. Note: must be fabricated and installed to meet ADA requirements, 36” clear opening, flush floor transition. See Architectural Plans. | Basco | Jacob Baldoc jbaldoc@bascoshowerdoor.com (513)283-9033 |  |
| Bathroom Sink Faucet | # Kohler – Honesty #K99760-4-CP. Single Handle Faucet, Polished Chrome. # (New replacement model number TBD) | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Soap Dish | Schluter Shelf - E Square. Item #SES1-D3-EB. 8” x 8”, Brushed Stainless Steel. Locate at showerhead end of shower, except in ADA Unit #3, (see plan). Install in grout joint at 42” to 54” h. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Toilet | Kohler – Highline Toilet Bowl #K-4199-O, Comfort Height, White, Elongated Bowl, 12” rough-in. Note: no caulk around toilet base. With Kohler Wellworth Toilet Tank #K4468-O. White, 1.6 gallon flush. Kohler – Brevia Elongated Seat #K-4774-O. White Polypropylene | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Toilet Paper Holder | Moen – Kyvos #BP3708CH, Polished Chrome Finish Toilet Paper Holder. Mounted at 28” high. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Towel Bar | Moen – Kyvos #BP-3724CH, Polished Chrome Finish 24” Towel Bar. Mounted at 48” high. When mounted over toilet, mount at 65” high. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Towel Ring | Moen – Kyvos #BP-3786CH, Polished Chrome Towel Ring. Mounted at 60” high to top of ring. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Robe Hook | Moen – Kyvos #BP-3703CH, Polished Chrome Double Robe Hook. Mounted at 70” high or to back of door as specified on plans. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Hall and Passage Door Hardware | Schlage - Solstice Satin Nickel Reversible Passage Door Lever. Item #1232274. (Hall and Passage, Walk-In Closets.) Model #563082. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Bedroom and Bath Door Hardware | Schlage - Solstice Satin Nickel Reversible Push Button Lock Privacy Door Lever. Item #1232273. (Bedroom and Bathroom.) Model # 563081. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Closet Door Hardware: Reversible | # Schlage - Solstice Satin Nickel Reversible Dummy Door Lever Item #1232275. (Closets) Model #563083. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Door Stops | # Lowes - #3632133, ReliaBilt Satin Nickel Rigid Door Stops. (Pkg. of 5) | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Unit Entry Door Wall Door Stop | # Lowes - #3703287, ReliaBilt Satin Chrome Bumper/Door Stop. | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Unit Entry Door Hardware & Lock | Dorma – J310 Series, Grade 2 Interconnected Locksets, Satin Chrome Finish, LC Lever Style. Rose Design B. Note: Verify Backset of 2-3/8” or 2-3/4” prior to ordering locks. | Sparks Hardware | Adam Bruener (859)572-0000 adam@sparkshardware.com |  |
| Unit Entry Door | Sparks Hardware – Unit Entry Doors- Baron HD Arch Top 2-Panel 18 gauge Unit Entry Frames- Timely A-Series adjustable frames-wood casing by others-primed finish. Apt. Entry Doors to have a 90 minute fire rating, per Architectural Plans. | Sparks Hardware | Adam Bruener (859)572-0000 adam@sparkshardware.com |  |
| Unit Door Signage | 4” x 6” oval Unit Identification Signage installed on Entry Doors with interior corridor access. Black & White Polyscribe with borders per standard design. Helvetica Medium Typeface. | Creative Blast | Paul Shoemaker (513)251-4177 creativeblast@fuse.net |  |
| Patio/ Balcony Doors | Match Existing Full Glass Lite Hinged Patio Door and Hardware to Building #3801. (Or equivalent style and grade.) |  |  |  |
| Passage Doors | # 2-Panel Arched Top Hollow Core Textured Molded Composite Interior Single Prehung Doors to match Existing Property Door. Size and Configuration per plans. Brushed Nickel Hinges. | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Pair of Closet doors | # Pair of 2-Panel Arched Top Hollow Core Textured Molded Composite Interior Single Prehung Doors to match Existing Property Doors. Size and Configuration per plans. Brushed Nickel Hinges | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Wall Base | # Cincinnati Orders: Lowes Item #73893, Model #623-PFJ12BD8. 3-1/4” x 9-1/16” x 12’. (8 Pack) # Note: Wall base used with LVT to be installed tight to subfloor. If required, ½” x ¾” Shoe Molding may be installed to cover any gaps. (not ¼ round) | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Door Casing | # EverTrue 2.25-in x 7-ft Interior Pine PFJ Type Window and Door Casing Item # 206592. Model # C 36670FJPMD | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Unit Vinyl Plank | CDC –Beauflor - Parkway Pro, Bark. 7.25” x 48” plank, 2mm thick, 12 mil wear layer. Glue Down LVT. | Axis Interior Systems Carpetland McSwain | Rep: Rod Seabolt(513)560-1040 rseabolt@cdcdist.com |  |
| Unit Bedroom Carpet | # Engineered Floors – Dreamweaver – Airwaves #E100-1945. Solution Dyed BCF Polyester. 6 lb., 7/16” rebond padding. Install Johnsonite Transition Strip #CE-29-A, Color: Moonrock with Track #MT-00-B for stretch-in carpet. | Axis Interior Systems Carpetland McSwain |  |  |
| Unit Paint | Sherwin Williams #SW-7056 Reserved White. Flat ProMar 200 on Walls, Eggshell on Bathroom Walls, and Semi-Gloss Super Paint on Doors and Trim | Sherwin Williams | Amber Lamb Amber.r.lamb@sherwin.com (513)430-1362 |  |
| Custom Closets for Master Bedrooms | California Closets. Only Master Bedroom to receive custom closet system. See layout provided by Rookwood. | California Closets | Kenton Dixon Email: kdixon@calclosets.com Cell: 317-610-1563 |  |
| Ventilated Wire Shelving | Rubbermaid Vented Wire Shelving. See plans for configuration. (Secondary bedrooms and closets) | Lowes |  |  |
| Outlet and Switch Plates | All outlet covers and light switch plates to be White Decora Style, per Quote #0009205. (Except Outlets in Kitchen Cabinets to be Dark Brown.) Note: when required, shallow outlet boxes may be used in cabinetry. (1-1/4” deep x 3-5/8” x 2-3/8”) | Mason Winlectric | Winlectric Aaron Biltz 513-615-4978 asbiltz@winlectric.com |  |
| USB Outlet for Kitchen Island | Maxxima USB Receptacle Outlet 4.8A USB C/A High Speed 4 USB wall charging outlet. Electrical AC replacement. Black Wall Plate included | Amazon | https://www.amazon.com/dp/B0B21Z116Q/ref=sspa_dk_detail_2?pd_rd_i=B0B21Z116Q&pd_rd_w=SZzNo&content-id=amzn1.sym.7446a9d1-25fe-4460-b135-a60336bad2c9&pf_rd_p=7446a9d1-25fe-4460-b135-a60336bad2c9&pf_rd_r=V3PZ9QZD26ASX55ZFPJE&pd_rd_wg=ue9Kp&pd_rd_r=25cc46b9-ffb5-443a-9491-998239e43df9&s=industrial&sp_csd=d2lkZ2V0TmFtZT1zcF9kZXRhaWw&th=1 |  |
| Balcony Exterior Wall Sconce | WAC Bandeau – WS-W2-1110-BK. 10” x 5” x 2-1/4”. Driverless LED, glass diffuser, set to 3000K. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Faux Wood Blinds | 2” White Faux Wood Blinds (Cordless). Smooth Finish. Place order 4-6 weeks prior to required installation. Include hold down brackets on Patio Door. | Sherwin Williams | Jonny Denny jonny.e.denny@sherwin.com (513)430-1362 |  |
|  | COMMON CORRIDORS |  |  |  |
| Stairwell Pendant | AFX - Cass 18” LED Round Pendant #CSSP36L30D1BKRND3. 18” dia., 36” high pendants suspended at various heights, Integrated LED, 120V, 108W, 90 CRI, 3000 K, 7800 delivered Lumens, Finish: Black, Frosted Acrylic Diffuser. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Corridor General Lighting | Satco - Nuvo #SATCO-S11827, 6.78” dia. x 1.02” high, 12W, 120V, White, Adjustable CCT set to 3500K, | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Corridor Wall Base | Johnsonite – Millwork Base #MW-29-K. 4” high, Ambassador, Color: Grey. (Corridors & Entry). Note - Install outside corners to be mitered and glued. |  |  |  |
| Corridor Carpet | Bentley - High Score II - Color: #400058, Ping. Affirma II Hardback Tile, 18” x 36” Tile Size. Installed in Brick Pattern. At Top Stair Tread install Stair Nosing. Kuberit Stair Nosing #KS-C-A1. 3mm or 5mm as required. Distributed by CDC. |  |  |  |
| Stairwell Carpet | Bentley - High Score II - Color: #400058, Ping. High Performance Broadloom Backing. Glue-down installation, or other as approved by owner. |  |  |  |
| Walk Off Carpet | Bentley Carpet - Rough Idea Shear - Color: 800117, Shape. 24” x 24” Carpet Tile with Affirma Hardback backing installed in quarter turn pattern. No transition strip required at transition to corridor carpet. |  |  |  |
| Paints | Corridor Wall Paint - Sherwin Williams #SW-7043, Worldly Grey. ProMar 200 Flat Finish. Corridor Ceiling Paint: Sherwin Williams #SW-7005 Pure White. ProMar 400 Latex, Flat. Stair Stringer Paint: Sherwin Williams Custom Color match to Johnsonite Grey Millwork Base. Latex, Satin Finish. Unit Entry Doors and Door Frames (Corridor Side Only): Sherwin Williams #SW-2819 Downing Slate. Pro Industrial Water Based Alkyd Urethane Enamel, Extra White Base, Semi-Gloss Finish. Note: prime with Extreme Bond Primer as needed. Building Entry Door and Frame (Both Sides): Sherwin Williams #SW-7076 Cyberspace. Pro Industrial Water Based Alkyd Urethane Enamel, Extra White Base, Semi-Gloss Finish. Use Extreme Bond Primer as required. |  |  |  |

Stairwell Base detail and Stair Nosing Detail from Kuberit.