Rookwood Properties – THE ENCLAVE III Master Specification List

TRADEMARK UNITS & PUBLIC SPACES

3800 Enclave Ave. Cincinnati, OH   45241

Property Manager: Beth Westgerdes. (513)769-8888. Sales@TheEnclaveCincinnati.com

Date: 9-9-24

| Item | Product Information | Vendor | Contact | Photo |
| --- | --- | --- | --- | --- |
| Cabinets (Kitchen & Bathroom) | Kountry Wood Products – Mission Door Style (Flush Panel Door), Onyx Finish on Maple. Note: All base cabinets to be 34.5” high, except ADA units. All Bathroom Vanity Cabinets to be 21” deep x 34.5” high. Kitchen Plans to include an 18” wide trash pull-out cabinet with 2 bins next to sink. Bathroom Vanities to have drawers. (30” vanities, drawers will be offset and sink located on side toward door.) | Nisbet Brower | Chris Fischer (513)619-7107 Chris.fischer@nisbetbrower.com |  |
| Kitchen Countertops | Quartz: Nisbet Brower Stock Color: “Segmanto”. Kitchen Countertops. Note: No quartz backsplash with tile backsplash. | Nisbet Brower | Chris Fischer (513)619-7107 chris.fischer@nisbetbrower.com |  |
| Kitchen Backsplash | Roca - Color Collection #U261, Tender Gray Matte Finish 3” x 6” x 6.3mm Subway Tile installed in horizontal brick pattern. Use 1/16” spacers. Grout – Mapei Ultracolor Plus FA, Color #02 Pewter. On exposed vertial edge, use Schluter – Scheine Trim #AE60, Satin Anodized Aluminum, 6mm. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Cabinet and Drawer Pulls | Richelieu Bow Pull #BP16338195. (Brushed Nickel) 128 mm center to center Bow Pull, 6” overall dimension. | Tamarack/ Richelieu | Rick Kunkemoeller (513)978-3755 (cell) rkunkemoeller@richelieu.com |  |
| Kitchen Faucet | # Kohler –Simplice Pull Down Kitchen Faucet #K-596-CP. Polished Chrome Finish. Install Handle on Right Side. Single hole installation - do not use escutcheon plate. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Kitchen Sink | Omicron - Pompano Series Undermount Stainless Steel Sink #SNKFAB3118. 30” x 18-1/8”, Inside dimensions: 28” x 16-1/8” x 9”). Install in 33” min. sink base, 30” if sides are cut. | Nisbet Brower | Chris Fischer (513)619-7107 Chris.fischer@nisbetbrower.com |  |
| Entry Pendant Fixture | AFX - Cole 24” LED Pendant. #COLP24L30D1SN. 24” x 16” x 15” high. Satin Nickel. 3000K, 36W, 4200 Lumens. Install with bottom edge at 7’-3” above floor. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Kitchen, Hall, Closet & Shower Light | Kichler #43848WHLED30T “Utilitarian” Flush Mount Ceiling Light, 7.5” wide, 1-1/4” high, 15W LED, direct mount to junction box. 5 year warrantee. Wet rated for showers. Alternate: Globalux LDL2-6-15-120-9-30-WH. 6” dia., 3000K. 15W. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Ceiling Fan for Master Bedrooms | Royal Pacific Fans – Europa #1004WW-E26L. Color: White. 15-7/8” high x 50” diameter, 12-1/2” to underside of blades, 3-Speed Reversible, (2) 9W A19 LED lamps included. WC-16 Wall Control. Install with stems so that underside of fan is at 8’0” above floor. | Ferguson | Rob Minardi 513-326-2986 cell: 513-484-5775 rob.minardi@ferguson.com |  |
| Pendant over Island /Sink | WAC Lighting - Level Pendant. #PD-85648-AL. 3500K, 1-1/2” x 4” x 48” LED Pendant. Aluminum with Acrylic Diffuser. Install with bottom edge at 6’-9” above floor. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Dishwasher | GE – Stainless Steel Front Control Dishwasher #GDF510PSRSS. 23-3/4” w x 24” d x 34” h. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Range | GE – Stainless Front, Freestanding Glass Top Electric Range #JB625RKSS. 29-7/8” w x 28” d x 47”h. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Range Cord | GE – 3 Wire, 4’, 40 Amp Cord #WX9X6. Or - 4 Wire, 4’, 40 Amp Cord #WX9X35. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Microwave | GE – Stainless Steel Over-the-Range Microwave Oven #JNM3163RJSS. 29-7/8” w x 15-7/8” d x 16-7/16” h. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Refrigerator | GE Stainless Steel Top Freezer, 17.5 cu. ft. Refrigerator with Icemaker. #GIE18GSNRSS. Note: Door comes with hinge on the left, if right swing is required order with code #M151 for installers to reverse the swing. 28” w x 30-1/2” d x 67-3/8” h. 30-5/8” wide with door open 90 degrees. No plastic water lines to icemakers – Use Copper or Pex Lines. | GE (Account #’s are specific to each Property) | Leann Sullivan (800)716-3795 PropertyManagement4@geappliances.com Online Order: www.GEAppliancesCustomerNet.com |  |
| Disposal | Lowes – InSinkErator #503602, Badger 5, ½ HP Disposal. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Hot Water Heaters | 40 Gallon Low Boy Water Heater to meet Architectural Specs. |  |  |  |
| Bathroom Vanity Light Fixture | Modernforms - Ice #WS-57927-CH. 27” wide x 3” high. 3500K, 27W, 2400 Lumens. Install at 81” to center of mounting box. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Framed Bathroom Mirror | 28” x 34” high Non-Beveled Mirror in 2” Frame Profile #313., Metal Z-Clip attached for vertical mounting. Delivery available. Install at 75” to top of frame. | Artonomy | Carrie Jennings/ Jayne Menke 513-651-2787 carrie@artonomyinc.com |  |
| Wall Tile at Shower Surround. | Hamilton Parker - Atlas Concorde - Square One - Odyssey, Matte Finish. 12” x 24” x 8mm Tile installed in vertical grid pattern. 1/8” grout joint. Grout – Mapei Ultracolor Plus FA, Color #02 Pewter. Use matching caulk #593315, Keracaulk S- Pewter. White Silicone Caulk at Tub joint. Install with Schluter - Schiene #AE-80, Satin Anodized Aluminum, edge band on vertical exposed tile edge. Note: Install 24” tile with Schluter trim on tiled shower pan curb. (Minimal grout joints) | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Shower Floor Tile | Panaria Home - Gray Range Muretto Linear Mosaic, Mesh Mounted, 12” x 28”. Mapei Ultracolor Plus FA, Color: #107, Iron. Install with Linear Slot Drain at showerhead end, Brushed Nickel Finish. Schluter Kerdi Shower Pan System. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Vanity Countertops and Sinks | Nisbet Brower Stock Quartz - 2cm #3003 GJ Quartz. 4” Backsplash and Sidesplash. Drilled for single hole faucet. | Nisbet Brower | Chris Fischer (513)619-7107 chris.fischer@nisbetbrower.com |  |
| Bathroom Undermount Sink | # Omicron 18” x 13” Rectangular Undermount White Porcelain Sink. # Alternate: MR Direct #U1913-White. Or Pelican Brand equal product is acceptable. | Nisbet Brower | Chris Fischer (513)619-7107 chris.fischer@nisbetbrower.com |  |
| Bathtub Faucet & Showerhead | # Kohler – Honesty Shower Head # #K-TS99-764-4-CP, Polished Chrome. # #KP8304-KS, Rite Temp Rough-In Valve with Stops. (Shower Units Only) To be installed at 42” high to center of shower vlave. # Kohler – Honesty Shower Head # #K-TS99-764-4-CP, Polished Chrome. Install at 80” high. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Shower Doors (Full Size 5’ Showers) | Basco Infinity Series – #4500-60, 76” Tall (Overall height from base of track to top of track) frameless ¼” thick bi-pass doors, Clear Glass, Silver Anodized Aluminum Frame, 2 Towel Bars Standard. | Basco | Jacob Baldoc jbaldoc@bascoshowerdoor.com (513)283-9033 |  |
| Bathroom Sink Faucet | # Kohler – Honesty #K99760-4-CP. Single Handle Faucet, Polished Chrome. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Soap Dish | Schluter Shelf - E Square. Item #SES1-D3-EB. 8” x 8”, Brushed Stainless Steel. | Hamilton Parker | Amanda Thom Athom@hamiltonparker.com 513-276-4840 |  |
| Toilet | Kohler – Highline Toilet Bowl #K-4199-O, Comfort Height, White, Elongated Bowl, 12” rough-in. With Kohler Wellworth Toilet Tank #K4468-O. White, 1.6 gallon flush. Kohler – Brevia Elongated Seat #K-4774-O. White Polypropylene | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Toilet Paper Holder | Moen – Kyvos #BP3708CH, Polished Chrome Finish Toilet Paper Holder. Mounted at 28” high. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Towel Bar | Moen – Kyvos #BP-3724CH, Polished Chrome Finish 24” Towel Bar. Mounted at 48” high. When mounted over toilet, mount at 65” high. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Towel Ring | Moen – Kyvos #BP-3786CH, Polished Chrome Towel Ring. Mounted at 60” high to top of ring. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Robe Hook | Moen – Kyvos #BP-3703CH, Polished Chrome Double Robe Hook. Mounted at 70” high or to back of door as specified on plans. | Ferguson | Ashley Smith Ashley.smith@ferguson.com Direct: 513-326-4746 Main office: (513)771-6000 |  |
| Hall and Passage Door Hardware | Schlage - Solstice Satin Nickel Reversible Passage Door Lever. Item #1232274. (Hall and Passage, Walk-In Closets.) Model #563082. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Bedroom and Bath Door Hardware | Schlage - Solstice Satin Nickel Reversible Push Button Lock Privacy Door Lever. Item #1232273. (Bedroom and Bathroom.) Model # 563081. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Closet Door Hardware: Reversible | # Schlage - Solstice Satin Nickel Reversible Dummy Door Lever Item #1232275. (Closets) Model #563083. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Door Stops | # Lowes - #3632133, ReliaBilt Satin Nickel Rigid Door Stops. (Pkg. of 5) | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Unit Entry Door Wall Door Stop | # Lowes - #3703287, ReliaBilt Satin Chrome Bumper/Door Stop. | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Unit Entry Door Hardware & Lock | Dorma – J310 Series, Grade 2 Interconnected Locksets, Satin Chrome Finish, LC Lever Style. Rose Design B. Note: Verify Backset of 2-3/8” or 2-3/4” prior to ordering locks. | Sparks Hardware | Adam Bruener (859)572-0000 adam@sparkshardware.com |  |
| Unit Entry Door | Sparks Hardware – Pioneer Industries #EH6, Honeycomb Core, Embossed, Steel Face 6 Panel Door, 18G, 3’-0” x 6’-8”. To include Viewers, Wall/Floor Stops, ADA Thresholds, Sweeps, Smoke Seals, Satin Chrome Spring Hinges, 16G-60 Min. Hollow Metal 3-Sided Door Frame. To be used with Dorma Interconnected Locksets. Primed finish to be painted accent color unique to each property. Doors to be sized per property and to meet fire ratings as required. Apt Entry Doors that open to a corridor must be 20 min rated. Apt Entry Doors that open to a stairwell must be 60 min rated. (unless walls have higher rating and then must match wall rating). Doors from Sparks to be ordered as “B” Label Doors, which are all 90 min. (price is same) | Sparks Hardware | Adam Bruener (859)572-0000 adam@sparkshardware.com |  |
| Unit Door Signage | 4” x 6” oval Unit Identification Signage installed on Entry Doors with interior corridor access. Black & White Polyscribe with borders per standard design. Helvetica Regular Typeface. | Creative Blast | Paul Shoemaker (513)251-4177 creativeblast@fuse.net |  |
| Louvered Doors | Masonite Louver over Louver (730) Primed MDF Louvered Door for use on Furnace Rooms. Sized per plan. | Lowes or other distributor | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Passage Doors | # ReliaBilt 6-Panel Hollow Core Textured Molded Composite Interior Single Prehung Doors. Size and Configuration per plans. # Typical item #17630. Brushed Nickel Hinges | Lowes | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Pair of 24” closet doors (Or other width as required) | # ReliaBilt 6-Panel Hollow Core Textured Molded Composite Reversible Interior French Door (Common: 48-in x 80-in; Actual: 53-in x 83.5-in) Item # 338796. Note: replace brass hinges with brushed nickel hinges. | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Wall Base | # Cincinnati Orders: Lowes Item #73893, Model #623-PFJ12BD8. 3-1/4” x 9-1/16” x 12’. (8 Pack) # Note: Wall base used with LVT to be installed tight to subfloor. If required, ½” x ¾” Shoe Molding may be installed to cover any gaps. (not ¼ round) | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Door Casing | # EverTrue 2.25-in x 7-ft Interior Pine PFJ Type Window and Door Casing Item # 206592. Model # C 36670FJPMD | Lowes Cinti/Dayton | Chad Cassidy Cell: 513-824-4279. Store: 513-598-7078. Email - chad.cassidy@store.lowes.com |  |
| Unit Vinyl Plank | CDC – LG Hausys – Savana #9135 Vintage. 12 mil wear layer, 6” x 48” Plank, 2.0 mm thick. Glue Down LVT. | Axis Interior Systems Carpetland McSwain | Brad Visconti 513-645-4279 Mobile: 513-905-9620. Bvisconti@axisinteriorsystems.com Rep: Rod Seabolt(513)560-1040 rseabolt@cdcdist.com |  |
| Unit Bedroom Carpet | # Engineered Floors – Dreamweaver – Airwaves #E100-1945. Solution Dyed BCF Polyester. 6 lb., 7/16” rebond padding. Install Johnsonite Transition Strip #CE-29-A, Color: Moonrock with Track #MT-00-B for stretch-in carpet. | Axis Interior Systems Carpetland McSwain | Brad Visconti 513-645-4279 Mobile: 513-905-9620. Bvisconti@axisinteriorsystems.com |  |
| Unit Paint | Sherwin Williams #SW-7056 Reserved White. Flat ProMar 200 on Walls, Eggshell on Bathroom Walls, and Semi-Gloss Super Paint on Doors and Trim | Sherwin Williams | Amber Lamb Amber.r.lamb@sherwin.com (513)430-1362 |  |
| Custom Closets for Bedrooms | California Closets. NOTE: All Bedrooms to have Custom Closet System. | California Closets | Kenton Dixon Email: kdixon@calclosets.com Cell: 317-610-1563 |  |
| Ventilated Wire Shelving | Rubbermaid Vented Wire Shelving | Lowes |  |  |
| Outlet and Switch Plates | All outlet covers and light switch plates to be White Decora Style, per Quote #0009205. (Except Outlets in Kitchen Cabinets to be Dark Brown.) Note: when required, shallow outlet boxes may be used in cabinetry. (1-1/4” deep x 3-5/8” x 2-3/8”) | Mason Winlectric | Winlectric Aaron Biltz 513-615-4978 asbiltz@winlectric.com |  |
| Balcony Exterior Wall Sconce | WAC Bandeau – WS-W2-1110-BK. 10” x 5” x 2-1/4”. Driverless LED, glass diffuser, set to 3000K. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Faux Wood Blinds | 2” White Faux Wood Blinds (Cordless). Smooth Finish. Place order 4-6 weeks prior to required installation. (Window Treatment for all Properties) | Sherwin Williams | Jonny Denny jonny.e.denny@sherwin.com (513)430-1362 |  |
|  | COMMON CORRIDORS |  |  |  |
| Exterior Sconce @ Building Entry | Kichler #KK4920BK. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Stairwell Pendant | AFX - Cass 18” LED Round Pendant #CSSP36L30D1BKRND3. 18” dia., 36” high pendants suspended at various heights, Integrated LED, 120V, 108W, 90 CRI, 3000 K, 7800 delivered Lumens, Finish: Black, Frosted Acrylic Diffuser. | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Corridor General Lighting | Satco - Nuvo #SATCO-S11827, 6.78” dia. x 1.02” high, 12W, 120V, White, Adjustable CCT set to 3500K, | Ferguson | Rob Minardi (513)771-6000 main (513)484-5775 cell Rob.minardi@ferguson.com |  |
| Corridor Wall Base | Johnsonite – Millwork Base #MW-29-K. 4” high, Ambassador, Color: Grey. (Corridors & Entry). Note - Install outside corners to be mitered and glued. |  |  |  |
| Corridor Carpet | Bentley - High Score II - Color: #400058, Ping. Affirma II Hardback Tile, 18” x 36” Tile Size. Installed in Brick Pattern. At Top Stair Tread install Stair Nosing. Kuberit Stair Nosing #KS-C-A1. 3mm or 5mm as required. Distributed by CDC. |  |  |  |
| Stairwell Carpet | Bentley - High Score II - Color: #400058, Ping. High Performance Broadloom Backing. Glue-down installation, or other as approved by owner. |  |  |  |
| Walk Off Carpet | Bentley Carpet - Rough Idea Shear - Color: 800117, Shape. 24” x 24” Carpet Tile with Affirma Hardback backing installed in quarter turn pattern. No transition strip required at transition to corridor carpet. |  |  |  |
| Paints | Corridor Wall Paint - Sherwin Williams #SW-7043, Worldly Grey. ProMar 200 Flat Finish. Corridor Ceiling Paint: Sherwin Williams #SW-7005 Pure White. ProMar 400 Latex, Flat. Stair Stringer Paint: Sherwin Williams Custom Color match to Johnsonite Grey Millwork Base. Latex, Satin Finish. Unit Entry Doors and Door Frames (Corridor Side Only): Sherwin Williams #SW-2819 Downing Slate. Pro Industrial Water Based Alkyd Urethane Enamel, Extra White Base, Semi-Gloss Finish. Note: prime with Extreme Bond Primer as needed. Building Entry Door and Frame (Both Sides): Sherwin Williams #SW-7076 Cyberspace. Pro Industrial Water Based Alkyd Urethane Enamel, Extra White Base, Semi-Gloss Finish. Use Extreme Bond Primer as required. |  |  |  |

Stairwell Base detail and Stair Nosing Detail from Kuberit.

EXTERIOR FINISHES ON UNIT BUILDINGS	- To be discussed

Brick:  Ironclay - Henry Brick - Samford Blend, standard morter.

Roof Shingles: IKO Cambridge - Color: Dual Black.

Railing: TimberTech - Radiance Rail Express System, Color: Classic Black.

Windows: White.

First Floor Vinyl Siding: Georgia Pacific – Clay, 5”.

Upper Level Vinyl Siding: Georgia Pacific – Tan, 4”.

Garage Vinyl Siding:  Georgia Pacific – Clay, 5”.

All soffits everywhere to be t4 white vinyl Solid soffit at porches and cornerS. Cv Soffit for eaves

All Siding .040 Vision Pro from Plygem

All fascia to be white aluminum.

Garage door jambs to be clay aluminum. (NOTE: this was incorrectly installed as white aluminum on original project)

All wood trim to be Paulowina 3/4” thick, smooth finish.

All flashings to be white alum except for deck flashing is grey PVC.

Paint Color on Metal Doors to Dumpster Enclosures:  Sherwin Williams #SW-6109, Hopsack. (Matches color of split face block).